SMART WATER TANK

Web Application

Pages  |  Workflow  |  How to Build

React.js  +  Node.js  +  MongoDB

2026

# 1. What Is This Website

The Smart Water Tank website is a full web application that lets you monitor and control your water tank from anywhere in the world — not just on your home WiFi. It is built with React.js on the frontend, Node.js on the backend, and MongoDB as the database.

Unlike the current ESP32 dashboard which only works at home, this website is hosted on the internet. The ESP32 sends data to a cloud server every few seconds, and the website reads from that server — so you can open it on your phone, laptop, or any device from anywhere.

Key difference: Current system — ESP32 hosts the website (home WiFi only). New system — cloud server hosts the website (accessible from anywhere in the world).

# 2. System Workflow

This is the complete flow of how the entire system works — from the sensor reading water level to you seeing it on screen and controlling the motor.

| # | Who / What | Action |
| --- | --- | --- |
| 1 | ESP32 Sensor | Reads water level from HC-SR04 ultrasonic sensor every 5 seconds |
| 2 | ESP32 Code | Converts distance to percentage and packages it as JSON data |
| 3 | ESP32 → Server | Sends JSON to the cloud server via HTTP POST or MQTT protocol |
| 4 | Server (Node.js) | Receives the data, validates it, and saves it to MongoDB database |
| 5 | Database (MongoDB) | Stores water level, motor state, timestamps, and user settings |
| 6 | Server API | Exposes REST API endpoints that the website can call to get data |
| 7 | Website (React.js) | Fetches data from server API every 2-3 seconds and shows on dashboard |
| 8 | User Action | User clicks motor ON button on the website |
| 9 | Website → Server | Website sends POST request to server: turn motor on |
| 10 | Server → ESP32 | Server sends command to ESP32 via MQTT or long-polling |
| 11 | ESP32 | Receives command, activates relay, motor turns on, confirms back to server |
| 12 | Server → Website | Server updates status, website reflects motor is now ON in real time |

# 3. Website Pages

The website has 6 main pages. Each page serves a specific purpose.

Page 1 — Login / Register

First page any user sees. Handles account creation and login.

- Login form with email and password

- Register form for new users

- Forgot password / reset password flow

- After login, user is redirected to their Dashboard

- JWT token stored in browser for authentication

Page 2 — Dashboard (Home)

Main page after login. Shows live overview of all tanks.

- Animated tank showing current water level with colour coding

- Motor status — Running or Stopped with pulsing indicator

- One-click motor ON / OFF buttons

- Current mode — Manual, Auto, or Schedule

- Live updating without page refresh (WebSocket or polling)

- Alert banner when tank is critically low or full

- Quick stats — WiFi signal, uptime, litres remaining

- If user has multiple tanks, shows a card for each tank

Page 3 — History & Analytics

Shows water level data over time with charts and stats.

- Line chart of water level for last 24 hours, 7 days, 30 days

- Motor ON/OFF event log with timestamps and reason

- Daily water usage in litres — bar chart

- Total litres used this week and this month

- Min and max level recorded each day

- Export data as CSV for spreadsheet use

Page 4 — Settings

All configuration for the tank and automation in one place.

- Calibration — set empty distance, full distance, tank capacity

- Auto Mode — set ON threshold and OFF threshold percentages

- Schedule — set motor ON time and OFF time with day selection

- Dry Run Protection — enable/disable, set wait time and min rise

- Physical Button — enable/disable, set web lock time

- Alert thresholds — set at what % to send notifications

- All settings saved immediately to server and pushed to ESP32

Page 5 — Alerts & Notifications

Manage how and when you get notified.

- View all past alerts — full or empty events with time

- Toggle browser notifications on or off

- Set custom notification thresholds (e.g. alert at 15% and 95%)

- Email notification settings

- Clear alert history

Page 6 — Account

User profile and tank management.

- Change name, email, password

- Add a new tank — give it a name, enter ESP32 device ID

- View all tanks linked to the account

- Remove a tank

- Logout

# 4. How to Build It

The website is built in three parts — Backend, Frontend, and connecting the ESP32. Build them in this order.

## Step 1 — Build the Backend (Server)

The backend is a Node.js server that runs on the internet. It handles all data storage and communication between the ESP32 and the website.

| Layer | Technology | Purpose |
| --- | --- | --- |
| Runtime | Node.js | Runs your JavaScript server code |
| Framework | Express.js | Makes building API routes easy |
| Database | MongoDB + Mongoose | Stores all water level data, users, settings |
| Auth | JWT + bcrypt | Secure login — passwords are hashed |
| Realtime | MQTT (Mosquitto) | ESP32 sends data here in real time |
| Hosting | Railway or Render | Free cloud hosting for the server |

API routes you need to build on the server:

- POST /auth/register  — create new user account

- POST /auth/login  — login, returns JWT token

- GET /api/tank/:id/data  — get latest water level and motor state

- GET /api/tank/:id/history  — get level history for charts

- POST /api/tank/:id/control  — send motor command to ESP32

- GET/POST /api/tank/:id/settings  — read and save all settings

- POST /api/esp32/data  — ESP32 posts data here every 5 seconds

## Step 2 — Build the React Frontend

The frontend is a React.js application. It calls your backend API to get data and show it on screen.

| Layer | Technology | Purpose |
| --- | --- | --- |
| Framework | React.js | Builds the user interface |
| Routing | React Router | Handles navigation between pages |
| API calls | Axios or Fetch | Calls your backend API |
| Charts | Recharts | Water level history graphs |
| State | React Context or Redux | Manages data shared across pages |
| Styling | Tailwind CSS | Makes styling fast and consistent |
| Hosting | Vercel or Netlify | Free hosting for the React app |

Folder structure for the React app:

src/

pages/  — Login, Dashboard, History, Settings, Alerts, Account

components/  — TankVisual, MotorCard, Chart, AlertBanner, Navbar

api/  — all fetch/axios calls to backend

context/  — AuthContext, TankContext

## Step 3 — Modify ESP32 Code

The ESP32 no longer hosts a website. Instead it just sends data to your server and listens for commands.

- Remove:  WebServer, handleRoot, and all HTML generation code

- Add:  MQTT client (PubSubClient library) to send data to server

- Add:  HTTP POST every 5 seconds to /api/esp32/data with level and motor state

- Add:  Subscribe to MQTT topic watertank/commands to receive motor commands from server

Install PubSubClient library in Arduino IDE. Use HiveMQ (free) or your own Mosquitto broker on Railway as the MQTT server.

# 5. Data Flow Summary

Everything that happens in the system follows one of two flows:

### Flow A — ESP32 sends data to website:

ESP32 → POST /api/esp32/data → Node.js Server → Saves to MongoDB → React Website fetches via GET /api/tank/data → Shows on screen

### Flow B — User controls motor from website:

User clicks ON → React sends POST /api/tank/control → Node.js Server → Publishes MQTT message → ESP32 receives command → Relay turns ON → ESP32 confirms back → Website updates

# 6. Tools You Need to Install

- Node.js  — download from nodejs.org (also installs npm)

- VS Code  — code editor, download from code.visualstudio.com

- MongoDB Compass  — visual database viewer, from mongodb.com/products/compass

- Postman  — for testing your API routes before connecting the frontend

- Git  — version control, from git-scm.com

- React (via Vite)  — run: npm create vite@latest to start a React project

You do not need to buy anything. Node.js, VS Code, MongoDB Atlas (free tier), Railway (free tier), and Vercel (free) are all free to use.

# 7. Advanced Features to Add

Once the basic website is working, these features will make it a complete, professional-grade product. Each feature is independent and can be added one by one.

## 7.1 Smart Automation Features

AI-Based Auto Fill Prediction

The system learns your daily water usage pattern and predicts when the tank will run empty before it actually does.

- Tracks how fast the water level drops each hour of the day

- Learns patterns — e.g. heavy usage at 7AM and 6PM

- Starts the motor 30 minutes before predicted empty time

- Sends you a prediction alert: Tank will be empty in 2 hours

- Gets smarter over time as more data is collected

- Built using simple moving average or ML model on the server

Smart Schedule — Weather Aware

Adjusts motor schedule based on weather — no point filling the tank if it is about to rain.

- Connects to OpenWeatherMap free API to get local forecast

- If heavy rain is predicted in next 6 hours, skips scheduled fill

- Shows weather forecast on dashboard alongside tank data

- Configurable — you set the rain threshold to skip

- Saves electricity and water by avoiding unnecessary motor runs

Water Conservation Mode

Automatically reduces how full the tank gets during water shortage periods.

- Set a conservation target — e.g. fill only to 60% instead of 90%

- Toggle conservation mode on/off from dashboard with one click

- Shows how many litres saved compared to normal mode

- Useful during summers or water rationing periods

## 7.2 Monitoring & Analytics Features

Advanced Analytics Dashboard

Deep insights into your water usage patterns over days, weeks, and months.

- Hourly water usage heatmap — see which hours consume most water

- Monthly comparison chart — this month vs last month

- Average daily fill cycles — how many times motor ran per day

- Water wastage detection — flags unusually long motor runs

- Cost estimation — calculates electricity cost of motor runs

- Downloadable PDF or CSV report for any date range

Tank Health Monitor

Tracks the health and performance of your pump and sensor over time.

- Motor run time tracker — total hours motor has run

- Dry run incident history — when and how many times dry run triggered

- Sensor accuracy log — flags if readings are inconsistent

- Maintenance reminder — alerts when motor has run for too many hours

- Pump performance score — based on how fast tank fills

Live Water Flow Rate

Calculates and displays how fast water is filling or draining in real time.

- Shows litres per minute filling rate when motor is ON

- Shows litres per minute drain rate during normal usage

- Detects abnormal drain rate — possible leak alert

- Flow rate history chart — see if pump is getting weaker over time

- Estimated time to fill from current level — shown on dashboard

## 7.3 Alert & Notification Features

Multi-Channel Notifications

Get alerted through multiple channels — not just browser notifications.

- WhatsApp message via Twilio or WhatsApp Cloud API

- Telegram bot notification — fastest and completely free

- Email alert with tank level snapshot

- SMS alert for critical situations (tank empty, dry run)

- Each alert type can be toggled on/off separately

- Set quiet hours — no alerts between 11PM and 7AM

Smart Alert Grouping

Prevents notification spam by grouping repeated alerts intelligently.

- If tank stays low for 30 minutes, only one alert is sent not 100

- Escalation alerts — first WhatsApp, then SMS if no response in 10 mins

- Alert acknowledgement — tap confirm in app to stop repeat alerts

- Weekly summary notification — every Sunday morning with usage stats

- Critical alerts bypass quiet hours — dry run always wakes you up

## 7.4 Multi-User & Security Features

Family & Team Access

Share tank monitoring with family members with different permission levels.

- Owner — full control, can change all settings

- Controller — can turn motor on/off but cannot change settings

- Viewer — can only see the dashboard, cannot control anything

- Invite members via email link

- Each member gets their own login

- Owner can remove any member at any time

- Activity log shows who did what and when

Security Features

Keeps your tank data and controls safe from unauthorized access.

- Two-factor authentication (2FA) via OTP on phone

- Login activity log — see all devices that logged in

- Auto logout after inactivity

- Rate limiting — prevents brute force password attempts

- HTTPS only — all data encrypted in transit

- Session management — remotely logout all devices

## 7.5 Multiple Tank Features

Multi-Tank Dashboard

Monitor and control multiple tanks from a single screen.

- Overview page showing all tanks as cards with live levels

- Each tank has its own separate settings and thresholds

- Group tanks — e.g. Ground Floor Tank, Terrace Tank

- Total water across all tanks shown as combined stat

- One click to switch between individual tank views

- Bulk actions — fill all tanks or stop all motors at once

Tank Linking — Overflow to Another Tank

Automatically pump water from one tank to another when overflow detected.

- Link two tanks — when Tank 1 is full, start filling Tank 2

- Prevents water overflow and wastage

- Fully configurable trigger level for each linked pair

- Shows flow direction visually on dashboard

## 7.6 UI & Experience Features

Fully Responsive & PWA

The website works perfectly on all screen sizes and can be installed like an app.

- PWA (Progressive Web App) — users can install it on phone home screen

- Works offline with cached data when internet is down

- Push notifications even when browser is closed

- Fully responsive — works on mobile, tablet, and desktop

- App-like smooth animations and transitions

Customisable Dashboard

Users can arrange and personalise their dashboard layout.

- Drag and drop widgets — put what matters most at the top

- Choose which stats to show on the main screen

- Custom tank name and icon for each tank

- Choose colour theme for the tank visual

- Compact mode for users who want more data on screen

Voice Assistant Integration

Control and check your tank using voice commands.

- Google Assistant — Hey Google, what is my water level

- Amazon Alexa — Alexa, turn on the water pump

- Built using IFTTT webhooks or direct Actions on Google SDK

- Responses include current level, motor state, and estimated fill time

Smart Water Tank Web Application Guide  |  2026